Supplementary Materials

for

Relationship between Perfluorooctanoate and Perfluorooctane Sulfonate Blood Concentrations in the General Population and Routine Drinking Water Exposure

Shiyi Zhang1, Qiyue Kang1, Hui Peng2, Mengyu Ding1, Fanrong Zhao1, Yuyin Zhou1, Zhaomin Dong3, Haifeng Zhang4, Min Yang4, and Jianying Hu*1

1 MOE Laboratory for Earth Surface Process, College of Urban and Environmental Sciences, Peking University, Beijing 100871, China

2 Department of Chemistry, University of Toronto, Toronto, Canada

3 School of Space and Environment, Beihang University, Beijing 100191, China

4 Key Laboratory of Aquatic Science and Technology, Research Center for Eco-Environmental Sciences, Chinese Academy of Sciences, Beijing, 100085, China.

Corresponding author:

Jianying Hu, College of Urban and Environmental Sciences, Peking University, Beijing 100871, China, TEL & FAX: 86-10-62765520, e-mail: hujy@urban.pku.edu.cn

## Chemicals and reagents. Standards including perfluoropentanoic acid (PFPeA), perfluorohexanoate (PFHxA), perfluoroheptanoate (PFHpA), perfluorooctanoate (PFOA), perfluorononanoate (PFNA), perfluorodecanoate (PFDA), perfluoroundecanoate (PFUnDA), perfluorododecanoate (PFDoDA), perfluorotridecanoate (PFTriDA), perfluorotetradecanoate (PFTeDA), perfluorohexanesulfonate (PFHxS), perfluoroheptanesulfonate (PFHpS), perfluorooctane sulfonate (PFOS), and perfluorodecanoatesulfonate (PFDS), and a mixture of mass-labelled perfluoroalkylcarboxylic acids (PFCAs) and isotopically-labelled perfluoroalkylsulfonates (PFSAs) standards including 13C2-PFHxA, 13C4-PFOA, 13C5-PFNA, 13C2-PFDA, 13C2-PFUnDA, 13C2-PFDoDA, 18O2-PFHxS, and 13C4-PFOS were purchased from Wellington Laboratories Inc. (Guelph, Ontario, Canada). HPLC-grade Methanol and methyl tert-butyl ether (MTBE) were purchased from Fisher Chemicals (New Jersey, USA). Tetra-n-butyl ammonium hydrogen sulfate (TBAS) and ammonia solution (NH4OH, 28%) were purchased from Alfa Aesar (Massachusetts, USA), and ammonium acetate (HPLC, 99.5%) was purchased from Dikma (Lake Forest, California, USA). A Millipore Milli-Q synthesis water purification system provided deionized water (Bedford, Massachusetts, USA).

## Sample extraction and cleanup. All water samples (500 mL) and procedure blanks were spiked with 50 μL of isotopically labeled internal standards (10 μg/L), then extracted using WAX cartridges (preconditioned with 6 mL of methanol containing 0.5% NH4OH, 6 mL of methanol, and 6 mL of ultrapure water at a ﬂow rate of 1–2 drops/s). After the cartridges were washed with 5 mL of ultrapure water and dried under a ﬂow of nitrogen, the cartridges were eluted with 6 mL of methanol containing 0.5% NH4OH. Extracts were dried under a gentle nitrogen stream and re-dissolved in 0.5 mL of methanol for UPLC–MS/MS analysis. Detailed instrumental conditions for analysis of PFAAs are described in Supplementary Materials.

# UPLC-ESI-MS/MS Analysis. All target chemicals were analyzed by a Premier XE tandem quadrupole mass spectrometer equipped with an electrospray ionization (ESI) source, coupled to an Acquity Ultra Performance LC (Waters, Milford, MA, USA). A C18 column (100 × 2.1 mm, 1.7 μm particle size) from Waters (Milford, Massachusetts, USA) was used for chromatographic separation at 40°C and a flow rate of 0.2 mL/min. For the sake of displacing the peaks caused by contamination from the UPLC, a C18 pre-column (50 mm × 2.1 mm, 1.7 μm particle size) (Milford, MA, USA) was used before the six-port valve of the UPLC. The injection volume was 10 μL. Methanol and water containing 5 mM ammonium acetate were utilized as mobile phases. Gradient conditions were initiated with 10% methanol followed by a linear increase to 75% methanol within 10 min. The proportion of methanol was raised to 100% at 15 min and held for 4 min before it was returned to 10% and the equilibration was kept for 4 min. Mass spectrometry was performed using a Premier XE tandem quadrupole mass spectrometer equipped with an electrospray ionization source and operated in the negative ion mode. The two most abundant multiple reaction monitoring (MRM) transitions, cone voltages, and collision energies were summarized in Table S2. Optimal MS conditions were as follows: source temperature, 110°C; desolvation temperature, 350°C; capillary voltage, 1.0 kV for the negative ion mode, 3.0 kV for the positive ion mode; desolvation gas flow, 800 L/h; and cone gas flow, 100 L/h.

# Quantitation and quality control. Analyte identification and quantification was completed using the MRM transitions at the expected retention time (within 2%) and ratio of the two product ions (within 20%). Standard injections were completed every 6 sample injections, followed by methanol injections to monitor potential instrument carry-over. Every 10 water or blood samples, a blank (deionized water) sample was analyzed to assess potential contamination during the entire procedure. To correct for possible loss of target compounds during sample preparation, potential matrix-induced changes to ionization, and variations in instrumental response, we used a set of stable isotopically labeled internal standards including 13C2-PFHxA, 13C4-PFOA, 13C5-PFNA, 13C2-PFDA, 13C2-PFUnDA, 13C2-PFDoDA, 18O2-PFHxS, and 13C4-PFOS (Table S2). Standard calibration curves of the target PFAAs were established with a concentration series of 0.1, 0.5, 1, 5, 10, 50, 100, and 500 μg/L. All calibration curves showed strong linearity (correlation coefficients >0.99) with good precision (RSD ≤ 5%). To minimize contamination, use and contact with Teflon-coated lab equipment was avoided and all equipment was rinsed regularly throughout the study with methanol and a mixture of methanol and dichloromethane. Recovery tests were conducted by spiking each target compound (1 ng/L water or 1 ng/mL blood), followed by the entire pretreatment procedure (n = 6). Since target compounds were not detected in the blanks, the LOQs or LODs were evaluated in matrix-spiked samples on the basis of signal-to-noise ratio (S/N = 10 or 3) of the transition reaction used for quantification.

# Calculation of the estimated daily intake using whole blood concentration. To convert body burdens to overall daily intake we used a simplified one-compartment toxicokinetic model described by Fromme et al. (2007) and Lorber and Egeghy (2011). First of all, the PFC concentrations in whole blood were converted to plasma concentrations (Cp) by multiplying by a factor of 2 (Kannan et al., 2004). At steady-state conditions, estimated daily intake (EDI) can then be described by the equation

EDI = k ∗ Vd ∗ Cp,

where Vd is the apparent volume of distribution (ml/kg) and k is the first-order rate constant elimination per day = 0.693/t1/2. For PFOA, a median half-life of 840 days was derived from the study of Bartell et al. (2010). The PFOS half-life of 1971 days is based on the results of Olsen et al. (2007). A human specific volume of distribution for PFOA and PFOS of 170 ml/kg b.w. and 230 ml/kg b.w. was used as described by Thompson et al. (2010).

## Table S1. Sampling information for drinking water from drinking water treatment plants (DWTPs) or individual homes.

| City | Total sample number | Number of DWTP | Sample number per DWTP | Sampling time | Source |
| --- | --- | --- | --- | --- | --- |
| Baoding | 10 | 10 | 1 | 2016/7/8(1), 2017/9/4(9)** | Haihe River |
| Beijing | 5 | 5 | 1 | 2016/9/6 | Haihe River |
| Binzhou | 3 | 3 | 1 | 2017/10/16 | Yellow River |
| Changzhou | 2 | 2 | 1 | 2017/12/7 | Taihu Lake |
| Chaohu* | 7 | 7 | 1 | 2017/5/17, 2018/3/3 | Chaohu Lake |
| Dalian | 4 | 4 | 1 | 2015/7/23 | Liao River |
| Dongguan | 10 | 10 | 1 | 2017/12/26 | Pearl River |
| Dongying | 3 | 3 | 1 | 2017/12/26 | Yellow River |
| Foshan | 8 | 8 | 1 | 2017/12/28 | Pearl River |
| Harbin | 4 | 2 | 1, 3 | 2015/7/2, 2016/9/6, 2017/3/28, 2017/12/7 | Songhua River |
| Heihe | 1 | 1 | 1 | 2017/12/7 | Heihe River |
| Jinan | 13 | 3 | 4(2), 5(1)** | 2015/9/23, 2016/8/28, 2017/1/11, 2017/7/2 | Yellow River |
| Nanjing | 12 | 3 | 4 | 2015/6/2, 2016/7/26, 2016/12/26, 2017/7/5 | Yangtze River |
| Lanzhou | 7 | 5 | 1(4), 3(1) | 2015/4/16(4), 2016/7/27, 2017/3/15, 2017/6/28 | Yellow River |
| Lianyungang | 3 | 3 | 1 | 2016/6/29 | Huai River |
| Shanghai | 4 | 4 | 1 | 2015/12/14 | Huangpu River |
| Shenyang | 6 | 2 | 3 | 2015/7/23, 2016/9/3, 2017/10/12 | Liao River |
| Shenzhen | 8 | 8 | 1 | 2016/12/2 | Pearl River |
| Shijiazhuang | 2 | 2 | 1 | 2016/7/13 | Haihe River |
| Tianjin | 4 | 1 | 4 | 2015/10/23, 2017/3/10, 2017/7/24, 2017/10/30 | Haihe River |
| Wuxi | 14 | 6 | 1(3), 3(1), 4(2) | 2015/5/14(3), 2016/9/1(3), 2017/1/25(3), 2017/5/19(3), 2017/12/7(2) | Taihu Lake |
| Zhengzhou | 4 | 2 | 2 | 2015/10/14, 2016/11/2 | Yellow River |
| Zhuzhou | 15 | 4 | 3(1), 4(3) | 2015/7/6(3), 2016/3/16(4), 2016/12/14(4), 2017/6/14(4) | Yangtze River |
| Zibo | 2 | 2 | 1 | 2017/10/16 | Yellow River |
| Changsha | 3 | tap water |  | 2018/1/10 | Yangtze River |
| Huhhot | 3 | tap water |  | 2018/1/15 | Yellow River |
| Mudanjiang | 3 | tap water |  | 2018/1/20 | Mudan River |
| Xi'an | 6 | tap water |  | 2018/1/25, 2018/4/25 | Yellow River |

* Cities in which blood samples were collected are in bold.

## ** The numbers in brackets indicate the number of DWTPs described by the corresponding water sample quantity or sampling time.

Table S2. Multiple reaction monitoring (MRM) transitions and mass spectrometry parameters of target compounds in the present study.

|  | Chemical | MRM | Cone voltage (V) | Collision energy (eV) | Surrogate |
| --- | --- | --- | --- | --- | --- |
|  | Formula | transition |  |  | Standard |
| PFPeA | C4F9CO2- | 263>269 a | 12 | 10 | 13C2-PFHxA |
|  |  | 263>119 |  | 15 |  |
| PFHxA | C5F11CO2- | 313>269 a | 13 | 10 | 13C2-PFHxA |
|  |  | 313>119 |  | 20 |  |
| PFHpA | C6F13CO2- | 363>319 a | 14 | 9 | 13C4-PFOA |
|  |  | 363>169 |  | 17 |  |
| PFOA | C7F15CO2- | 413>369 a | 15 | 10 | 13C4-PFOA |
|  |  | 413>169 |  | 18 |  |
| PFNA | C8F17CO2- | 463>419 a | 16 | 11 | 13C5-PFNA |
|  |  | 463>219 |  | 17 |  |
| PFDA | C9F19CO2- | 513>469 a | 20 | 12 | 13C2-PFDA |
|  |  | 513>219 |  | 16 |  |
| PFUnDA | C10F21CO2- | 563>519 a | 20 | 13 | 13C2-PFUnDA |
|  |  | 563>169 |  | 20 |  |
| PFDoDA | C11F23CO2- | 613>569 a | 23 | 12 | 13C2-PFDoDA |
|  |  | 613>169 |  | 25 |  |
| PFTriDA | C12F25CO2- | 663>619 a | 23 | 12 | 13C2-PFDoDA |
|  |  | 663>269 |  | 26 |  |
| PFBS | C4F9SO3- | 299>80a | 52 | 40 | 18O2-PFHxS |
|  |  | 299>99 |  | 32 |  |
| PFHxS | C6F13SO3- | 399>80 a | 52 | 40 | 18O2-PFHxS |
|  |  | 399>99 |  | 32 |  |
| PFHpS | C7F15SO3- | 449>80 a | 50 | 45 | 13C4-PFOS |
|  |  | 449>99 |  | 35 |  |
| PFOS | C8F17SO3- | 499>80a | 55 | 50 | 13C4-PFOS |
|  |  | 499>99 |  | 40 |  |
| PFDS | C8F21SO3- | 598>80a | 42 | 40 | 13C4-PFOS |
|  |  | 598>99 |  | 34 |  |

a MRM transition used for quantitation.

Table S3. Recoveries, LODs and LOQs of target compounds in spiked materials.

|  |  | Water Sample |  | Blood Sample |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Recovery,% | LOD, ng/L | LOQ, ng/L | Recovery, % | LOD, ng/L | LOQ, ng/mL |
| PFPeA | 92±4 | 0.06 | 0.2 | 91±17 | 0.054 | 0.18 |
| PFHxA | 81±3 | 0.06 | 0.2 | 86 ± 6 | 0.036 | 0.12 |
| PFHpA | 115±2 | 0.06 | 0.2 | 63 ± 10 | 0.015 | 0.05 |
| PFOA | 110±3 | 0.03 | 0.1 | 78 ± 4 | 0.012 | 0.04 |
| PFNA | 120±8 | 0.015 | 0.05 | 79 ± 8 | 0.012 | 0.04 |
| PFDA | 116±1 | 0.015 | 0.05 | 79 ± 11 | 0.012 | 0.04 |
| PFUnDA | 88±15 | 0.03 | 0.1 | 69 ± 5 | 0.006 | 0.02 |
| PFDoDA | 95±5 | 0.03 | 0.1 | 67 ± 10 | 0.012 | 0.04 |
| PFTriDA | 75±3 | 0.06 | 0.2 | 61 ± 4 | 0.018 | 0.06 |
| PFBS | 84±1 | 0.015 | 0.05 | 77± 2 | 0.006 | 0.02 |
| PFHxS | 82±5 | 0.015 | 0.05 | 77 ± 6 | 0.021 | 0.07 |
| PFHpS | 83±3 | 0.015 | 0.05 | 79 ± 6 | 0.012 | 0.04 |
| PFOS | 79±5 | 0.015 | 0.05 | 66 ± 9 | 0.03 | 0.1 |
| PFDS | 85±2 | 0.03 | 0.1 | 63 ± 6 | 0.006 | 0.02 |

# Table S4. Comparisons of PFOS (a) and PFOA (b) levels (mean, ng/L) in drinking water samples from different studies.

|  | PFOA |  |  | PFOS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 2002-2006a | 2006-2008b | 2015-2018 | 2002-2006a | 2006-2008b | 2015-2018 |
| Beijing | 3.88 | 0.44 | 9.31 | 0.54 | 0.04 | 3.08 |
| Dalian | 0.54 |  | ND | 0.35 |  | 0.85 |
| Harbin | 0.12 |  | 0.58 | 0.12 |  | 0.06 |
| Nanjing |  | 5.90 | 8.50 |  | 0.94 | 1.10 |
| Shanghai |  | 78.00 | 4.24 |  | 7.60 | 9.09 |
| Shenyang | 1.60 | 0.79 | 4.56 | 1.85 | 0.39 | 0.25 |
| Shenzhen | 45.95 | 1.00 | 2.37 | 14.85 | 11.00 | 7.45 |
| Xi'an | ND |  | 0.39 | 0.40 |  | 0.44 |

a Jin et al., 2009; b Mak et al., 2009.

# Table S5. Perfluorinated compounds concentrations in whole blood samples from 13 cities of China (ng/mL).

| City (No. of samples) |  | PFHxA | PFHpA | PFOA | PFNA | PFDA | PFUnDA | PFDoDA | PFTriDA | PFHxS | PFHpS | PFOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chaohu(48) | Frequency | 6% | 4% | 100% | 100% | 100% | 98% | 27% | 27% | 90% | 18% | 100% |
|  | Mean | 0.02 | 0.04 | 2.54 | 0.50 | 0.64 | 0.20 | 0.03 | 0.05 | 0.12 | 0.04 | 1.88 |
|  | GM | 0.02 | 0.03 | 2.25 | 0.40 | 0.45 | 0.15 | 0.02 | 0.04 | 0.06 | 0.03 | 1.55 |
|  | Max. | 0.05 | 0.11 | 6.25 | 1.55 | 4.02 | 0.86 | 0.10 | 0.19 | 0.51 | 0.27 | 7.55 |
| Changsha(100) | Frequency | 15% | 20% | 100% | 100% | 100% | 100% | 68% | 52% | 97% | 0% | 100% |
|  | Mean | 0.03 | 0.05 | 1.07 | 0.49 | 0.42 | 0.48 | 0.07 | 0.07 | 0.32 | - | 2.92 |
|  | GM | 0.03 | 0.04 | 0.87 | 0.40 | 0.36 | 0.42 | 0.04 | 0.05 | 0.22 | - | 2.34 |
|  | Max. | 0.31 | 0.20 | 8.13 | 2.33 | 1.20 | 1.26 | 1.38 | 0.26 | 1.24 | - | 10.43 |
| Hohhot(50) | Frequency | 8% | 4% | 100% | 100% | 100% | 96% | 8% | 8% | 77% | 23% | 100% |
|  | Mean | 0.04 | 0.05 | 2.84 | 0.29 | 0.20 | 0.24 | 0.03 | 0.07 | 0.20 | 0.05 | 2.98 |
|  | GM | 0.03 | 0.04 | 1.35 | 0.21 | 0.14 | 0.14 | 0.02 | 0.03 | 0.06 | 0.03 | 2.25 |
|  | Max. | 0.82 | 0.64 | 63.67 | 1.84 | 1.06 | 2.27 | 0.70 | 1.42 | 1.24 | 0.25 | 11.46 |
| Jinan(50) | Frequency | 6% | 4% | 100% | 100% | 100% | 100% | 0% | 0% | 100% | 0% | 100% |
|  | Mean | 0.03 | 0.04 | 5.34 | 0.52 | 0.31 | 0.32 | - | - | 0.24 | - | 2.59 |
|  | GM | 0.02 | 0.04 | 4.40 | 0.43 | 0.25 | 0.27 | - | - | 0.15 | - | 2.00 |
|  | Max. | 0.11 | 0.33 | 24.88 | 1.55 | 0.91 | 1.11 | - | - | 2.04 | - | 8.64 |
| Mudanjiang(100) | Frequency | 14% | 2% | 100% | 100% | 100% | 100% | 49% | 62% | 100% | 46% | 100% |
|  | Mean | 0.03 | 0.03 | 1.46 | 0.47 | 0.23 | 0.31 | 0.03 | 0.08 | 0.27 | 0.09 | 3.29 |
|  | GM | 0.02 | 0.03 | 1.24 | 0.40 | 0.20 | 0.26 | 0.03 | 0.06 | 0.17 | 0.05 | 2.48 |
|  | Max. | 0.07 | 0.07 | 5.75 | 1.49 | 0.56 | 0.90 | 0.10 | 0.32 | 1.16 | 0.77 | 14.04 |
| Shenyang(50) | Frequency | 6% | 4% | 100% | 100% | 100% | 100% | 59% | 69% | 98% | 98% | 100% |
|  | Mean | 0.02 | 0.04 | 3.58 | 0.81 | 0.40 | 0.48 | 0.04 | 0.11 | 0.89 | 2.19 | 9.41 |
|  | GM | 0.02 | 0.04 | 2.99 | 0.72 | 0.35 | 0.41 | 0.03 | 0.08 | 0.48 | 1.16 | 6.82 |
|  | Max. | 0.05 | 0.21 | 20.63 | 2.55 | 1.01 | 1.09 | 0.13 | 0.40 | 3.60 | 9.09 | 51.72 |
| Wuxi(50) | Frequency | 12% | 31% | 100% | 100% | 100% | 100% | 92% | 92% | 100% | 92% | 100% |
|  | Mean | 0.03 | 0.06 | 8.65 | 1.97 | 2.49 | 1.69 | 0.17 | 0.21 | 2.63 | 0.18 | 14.80 |
|  | GM | 0.02 | 0.05 | 7.61 | 1.71 | 1.99 | 1.38 | 0.11 | 0.16 | 2.31 | 0.14 | 11.55 |
|  | Max. | 0.07 | 0.29 | 35.73 | 6.18 | 8.78 | 6.58 | 1.27 | 1.01 | 6.77 | 0.76 | 60.08 |
| Shanghai(100) | Frequency | 2% | 15% | 100% | 97% | 98% | 92% | 58% | 0% | 94% | 1% | 100% |
|  | Mean | 0.02 | 0.04 | 6.69 | 1.08 | 0.82 | 0.48 | 0.18 | 0.03 | 0.45 | 0.02 | 3.60 |
|  | GM | 0.02 | 0.04 | 4.22 | 0.34 | 0.40 | 0.19 | 0.06 | 0.03 | 0.24 | 0.02 | 2.25 |
|  | Max. | 0.05 | 0.15 | 44.84 | 14.66 | 6.51 | 6.09 | 3.32 | 0.03 | 4.28 | 0.14 | 19.69 |
| Zhengzhou(50) | Frequency | 0% | 0% | 98% | 98% | 98% | 98% | 4% | 0% | 98% | 50% | 98% |
|  | Mean | - | - | 1.42 | 0.19 | 0.11 | 0.11 | 0.02 | 0.03 | 0.14 | 0.12 | 1.48 |
|  | GM | - | - | 1.18 | 0.16 | 0.09 | 0.10 | 0.02 | 0.03 | 0.10 | 0.06 | 1.23 |
|  | Max. | - | - | 4.58 | 0.41 | 0.31 | 0.30 | 0.04 | 0.03 | 0.72 | 0.80 | 4.00 |
| Zhuzhou(90) | Frequency | 33% | 14% | 100% | 100% | 100% | 99% | 73% | 78% | 96% | 47% | 100% |
|  | Mean | 0.05 | 0.04 | 1.67 | 0.77 | 0.62 | 0.84 | 0.07 | 0.11 | 0.29 | 0.17 | 4.36 |
|  | GM | 0.04 | 0.04 | 1.51 | 0.68 | 0.54 | 0.65 | 0.05 | 0.09 | 0.13 | 0.07 | 3.61 |
|  | Max. | 0.39 | 0.19 | 3.94 | 2.67 | 1.95 | 7.53 | 0.24 | 0.34 | 3.88 | 2.97 | 19.25 |
| Nanjing(60) | Frequency | 30% | 20% | 100% | 100% | 100% | 100% | 100% | 84% | 98% | 0% | 100% |
|  | Mean | 0.05 | 0.07 | 6.14 | 2.14 | 2.53 | 1.51 | 0.14 | 0.15 | 0.43 | - | 10.05 |
|  | GM | 0.04 | 0.05 | 5.64 | 1.88 | 2.16 | 1.29 | 0.12 | 0.11 | 0.37 | - | 8.36 |
|  | Max. | 0.32 | 1.10 | 13.05 | 6.31 | 7.69 | 4.44 | 0.43 | 0.44 | 1.19 | - | 31.18 |
| Harbin(49) | Frequency | 92% | 29% | 100% | 100% | 100% | 100% | 90% | 100% | 96% | 0% | 100% |
|  | Mean | 0.21 | 0.06 | 2.14 | 1.07 | 0.59 | 0.58 | 0.09 | 0.13 | 0.19 | 0.02 | 4.35 |
|  | GM | 0.15 | 0.05 | 1.85 | 0.84 | 0.50 | 0.49 | 0.07 | 0.10 | 0.13 | 0.02 | 3.38 |
|  | Max. | 1.01 | 0.23 | 8.03 | 3.78 | 1.80 | 1.55 | 0.33 | 0.39 | 0.96 | 0.02 | 16.64 |
| Xi'an(60) | Frequency | 0% | 0% | 100% | 95% | 80% | 87% | 0% | 0% | 67% | 0% | 100% |
|  | Mean | - | - | 1.32 | 0.15 | 0.11 | 0.10 | - | - | 0.09 | - | 1.68 |
|  | GM | - | - | 1.16 | 0.11 | 0.07 | 0.07 | - | - | 0.02 | - | 1.18 |
|  | Max. | - | - | 3.30 | 0.48 | 0.53 | 0.29 | - | - | 0.60 | - | 10.35 |

# Table S6. Drinking water concentrations (ng/L) for all compounds.

|  |  | PFPeA | PFHxA | PFHpA | PFOA | PFNA | PFDA | PFUnDA | PFDoDA | PFBS | PFHxS | PFHpS | PFOS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Baoding | Mean | - | 0.27 | 0.03 | 0.56 | 0.22 | 0.05 | 0.01 | - | - | 0.08 | - | 0.06 |
|  | Std | - | 0.15 | 0.10 | 0.28 | 0.14 | 0.06 | 0.03 | - | - | 0.11 | - | 0.12 |
| Beijing | Mean | 0.91 | 3.94 | 1.88 | 9.31 | 0.59 | 0.09 | 0.03 | - | 0.34 | 0.13 | 0.07 | 3.08 |
|  | Std | 0.71 | 5.00 | 2.48 | 12.22 | 0.63 | 0.08 | 0.03 | - | 0.40 | 0.18 | 0.10 | 4.38 |
| Binzhou | Mean | 1.25 | 1.38 | 1.20 | 6.72 | 0.51 | 0.06 | 0.05 | 0.01 | 3.22 | 18.05 | - | 2.25 |
|  | Std | 1.09 | 1.20 | 1.09 | 8.11 | 0.46 | 0.06 | 0.05 | 0.02 | 2.80 | 16.52 | - | 1.95 |
| Changzhou | Mean | 0.52 | 0.70 | 0.84 | 12.09 | 0.64 | 0.25 | 0.14 | 0.09 | 7.59 | 12.24 | 0.97 | 5.73 |
|  | Std | 0.01 | 0.05 | 0.26 | 2.29 | 0.32 | 0.08 | 0.06 | 0.01 | 1.35 | 14.65 | 1.38 | 1.06 |
| Chaohu | Mean | 2.02 | 2.32 | 1.96 | 10.56 | 1.62 | 0.50 | 0.18 | 0.02 | 7.81 | 29.53 | 0.04 | 3.33 |
|  | Std | 0.28 | 0.37 | 0.36 | 1.47 | 0.26 | 0.09 | 0.04 | 0.04 | 1.52 | 17.22 | 0.11 | 0.69 |
| Dalian | Mean | 0.44 | 0.83 | 0.95 | 0.02 | 1.11 | 0.30 | 0.10 | - | 0.09 | 0.05 | - | 0.85 |
|  | Std | 0.10 | 0.15 | 0.13 | - | 0.42 | 0.06 | 0.01 | - | 0.04 | 0.10 | - | 0.35 |
| Dongguan | Mean | 1.15 | 2.07 | 0.78 | 3.13 | 0.69 | 0.32 | 0.18 | 0.02 | 5.60 | 1.29 | 1.19 | 26.65 |
|  | Std | 0.13 | 0.28 | 0.19 | 0.72 | 0.13 | 0.08 | 0.04 | 0.04 | 1.61 | 0.42 | 0.46 | 9.55 |
| Dongying | Mean | 1.58 | 2.14 | 1.17 | 7.13 | 0.51 | 0.14 | 0.10 | - | 2.82 | 15.27 | - | 4.37 |
|  | Std | 1.13 | 1.45 | 1.01 | 5.25 | 0.28 | 0.04 | 0.10 | - | 2.57 | 18.14 | - | 5.28 |
| Foshan | Mean | 0.19 | 0.32 | 0.91 | 45.50 | 0.21 | 0.17 | 0.12 | 0.03 | 0.48 | 0.09 | 0.06 | 5.02 |
|  | Std | 0.22 | 0.31 | 1.23 | 67.97 | 0.08 | 0.07 | 0.07 | 0.05 | 0.34 | 0.13 | 0.17 | 4.79 |
| Harbin | Mean | 0.05 | 0.18 | 0.22 | 0.58 | 0.36 | 0.09 | 0.04 | - | 0.03 | 0.03 | 0.03 | 0.05 |
|  | Std | 0.06 | 0.12 | 0.16 | 0.11 | 0.13 | 0.06 | 0.04 | - | 0.03 | 0.05 | 0.06 | 0.05 |
| Heihe | Mean | - | - | 0.10 | 0.02 | 0.21 | 0.13 | 0.07 | - | - | - | - | 0.01 |
|  | Std | - | - | - | - | - | - | - | - | - | - | - | - |
| Hohhot | Mean | 0.31 | 0.02 | 0.06 | 0.50 | 0.03 | - | 0.03 | - | 0.19 | - | - | 0.07 |
|  | Std | 0.53 | 0.01 | 0.05 | 0.30 | 0.03 | - | 0.05 | - | 0.33 | - | - | 0.12 |
| Jinan | Mean | 1.13 | 3.70 | 1.84 | 13.24 | 4.54 | 0.31 | 0.00 | 0.00 | 1.32 | 4.89 | 0.04 | 5.01 |
|  | Std | 1.03 | 5.90 | 2.77 | 15.45 | 6.23 | 0.32 | 0.02 | 0.01 | 1.40 | 6.63 | 0.09 | 6.20 |
| Lanzhou | Mean | - | 0.03 | 0.03 | 0.85 | 0.19 | 0.07 | 0.02 | 0.05 | 0.54 | 0.18 | 0.01 | 0.21 |
|  | Std | - | 0.08 | 0.09 | 1.68 | 0.25 | 0.09 | 0.05 | 0.09 | 1.42 | 0.41 | 0.03 | 0.48 |
| Lianyunguang | Mean | 3.24 | 56.81 | 0.87 | 61.42 | 19.72 | 3.29 | 0.07 | - | - | 1.01 | 0.03 | 186.17 |
|  | Std | 2.85 | 26.96 | 0.61 | 20.77 | 17.25 | 2.85 | 0.13 | - | - | 0.76 | 0.03 | 49.65 |
| Mudanjiang | Mean | 0.15 | 0.08 | 0.47 | 0.82 | 0.37 | - | 0.04 | 0.04 | - | - | - | - |
|  | Std | 0.17 | 0.06 | 0.15 | 0.08 | 0.04 | - | 0.04 | 0.07 | - | - | - | - |
| Nanjing | Mean | 0.41 | 0.83 | 0.98 | 8.50 | 3.20 | 0.12 | 0.02 | - | 4.54 | 1.14 | 0.15 | 1.09 |
|  | Std | 0.45 | 0.68 | 1.40 | 5.77 | 5.22 | 0.17 | 0.03 | - | 7.26 | 0.94 | 0.44 | 1.03 |
| Shnaghai | Mean | 3.96 | 27.08 | 4.65 | 4.24 | 1.52 | 1.35 | 0.35 | - | 2.46 | 6.13 | 0.04 | 9.09 |
|  | Std | 3.69 | 31.27 | 4.42 | 4.89 | 1.46 | 1.39 | 0.41 | - | 0.90 | 6.43 | 0.08 | 9.27 |
| Shenzhen | Mean | 0.44 | 0.18 | 0.42 | 2.37 | 0.30 | 0.37 | 0.07 | - | 0.74 | 7.62 | 1.56 | 7.45 |
|  | Std | 0.45 | 0.10 | 0.29 | 1.87 | 0.21 | 0.20 | 0.10 | - | 0.65 | 20.87 | 4.22 | 5.18 |
| Shenyang | Mean | 1.17 | 1.20 | 0.65 | 4.56 | 0.49 | 0.16 | 0.01 | - | 0.13 | 0.04 | - | 0.25 |
|  | Std | 0.99 | 0.95 | 0.34 | 3.58 | 0.20 | 0.09 | 0.03 | - | 0.11 | 0.06 | - | 0.19 |
| Shijiazhuang | Mean | 0.25 | 5.94 | 2.18 | 25.84 | 8.71 | 0.74 | - | - | - | 0.04 | 2.79 | 14.70 |
|  | Std | 0.36 | 8.40 | 2.08 | 34.93 | 1.59 | 0.56 | - | - | - | 0.05 | 3.32 | 8.53 |
| Tianjin | Mean | 0.47 | 7.05 | 0.95 | 2.30 | 0.64 | 0.20 | 0.08 | 0.06 | 0.28 | 0.06 | - | 0.76 |
|  | Std | 0.40 | 13.20 | 1.09 | 1.68 | 0.60 | 0.16 | 0.07 | 0.11 | 0.29 | 0.08 | - | 0.31 |
| Wuxi | Mean | 2.46 | 13.60 | 3.49 | 27.79 | 3.08 | 0.83 | 0.20 | 0.06 | 7.05 | 36.04 | 0.10 | 7.52 |
|  | Std | 1.61 | 17.55 | 2.25 | 15.36 | 2.77 | 0.68 | 0.20 | 0.09 | 4.51 | 33.52 | 0.19 | 7.39 |
| Xi'an | Mean | - | 0.01 | 0.10 | 0.39 | 0.10 | 0.07 | 0.08 | 0.02 | 0.03 | - | - | 0.44 |
|  | Std | - | 0.02 | 0.07 | 0.07 | 0.03 | 0.06 | 0.12 | 0.05 | 0.08 | - | - | 0.24 |
| Changsha | Mean | 0.05 | 0.08 | 0.18 | 0.53 | 0.23 | 0.06 | 0.08 | 0.01 | - | - | - | 0.14 |
|  | Std | 0.08 | 0.04 | 0.03 | 0.04 | 0.04 | 0.07 | 0.02 | 0.02 | - | - | - | 0.23 |
| Zhengzhou | Mean | 0.13 | 0.28 | 0.08 | 0.89 | 0.09 | - | - | - | 0.26 | 0.58 | - | 0.11 |
|  | Std | 0.15 | 0.08 | 0.10 | 0.80 | 0.10 | - | - | - | 0.31 | 1.00 | - | 0.10 |
| Zhuzhou | Mean | 0.17 | 1.56 | 0.48 | 2.11 | 0.47 | 0.09 | 0.02 | - | 0.30 | 0.01 | - | 0.33 |
|  | Std | 0.22 | 1.69 | 0.21 | 2.02 | 0.36 | 0.09 | 0.05 | - | 0.51 | 0.03 | - | 0.31 |
| Zibo | Mean | 1.95 | 2.00 | 1.82 | 15.89 | 0.69 | 0.16 | 0.05 | - | 2.96 | 13.38 | - | 2.11 |
|  | Std | 1.20 | 1.74 | 1.68 | 9.78 | 0.28 | 0.06 | 0.07 | - | 3.65 | 18.93 | - | 2.13 |

-: undetected data.

# Figure S1. Linear regression of daily intake from drinking water versus daily intake EDI for PFOA.