Supplementary Materials

Exposure Assessment of PCDD/Fs for the Population Living in the Vicinity of Municipal Waste Incinerator: Additional Exposure via Local Vegetable Consumption

Yujie Ben, Tong Li, Yi Wan, Zhaomin Dong, and Jianying Hu*

MOE Laboratory for Earth Surface Process, College of Urban and Environmental Sciences, Peking University, Beijing 100871, China

Corresponding author:

Jianying Hu,

College of Urban and Environmental Sciences

Peking University

Beijing 100871, China

TEL & FAX: 86-10-62765520

E-mail: hujy@urban.pku.edu.cn

Text, tables and figures addressing: (1) chemicals and regents; (2) analytical method; (3) a summary of PCDD/Fs in vegetation collected near MSWIs in other studies; (4) PCDD/F concentrations in other types of commercial food obtained from the market; (5) human exposure to dioxin via vegetable consumption in other studies; (6) Prediction of the distribution of annual average concentration of PCDD/Fs by the Gaussian model (Environmental  Assessment  Center  of  Peking  University, 2003); (7) PCDD/F congener patterns in atmospheric samples, soil samples and vegetable samples grown in the vicinity of the MSWI.

Chemicals and Regents. 13C-labeled surrogate standards: DF-LCS-B (including 13C12-2378-T4CDD, 13C12-12378-P5CDD, 13C12-123478-H6CDD, 13C12-123678-H6CDD, 13C12-123789-H6CDD, 13C12-1234678-H7CDD, 13C12-2378-T4CDF, 13C12-12378-P5CDF, 13C12-23478-P5CDF, 13C12-123478-H6CDF, 13C12-123678-H6CDF, 13C12-123789-H6CDF, 13C12-234678-H6CDF, 13C12-1234678-H7CDF, 1 μg/mL; 13C12-OCDD and 13C12-OCDF, 2 μg/mL), injection standards: DF-IS-E (including 13C12-1368-T4CDD and 13C12-1234689- H7CDF, 1 μg/mL), and calibration standard solutions: CS2, CS4, CS6, CS8 and CS10 of DF-CVS-A10 were obtained from Wellington Laboratories (Guelph, Canada). Hexane, dichloromethane and toluene were ultra residues analysis grade and purchased from J.T. Baker Co. Ltd. (Phillipsburg, USA). Sodium sulfates was pesticide analysis grade and purchased from ANPEL Laboratory Technologies Inc. (Shanghai, China). Silica gel and activated carbon dispersed silica gel were purchased from Kanto Chemical Co. Inc (Tokyo, Japan).

Analytical Method. Meat, fruits and vegetables were washed by distilled water, chopped and freeze-dried. Eggs, shrimp and spiral shell were shelled and freeze-dried. After spiked with a mixture of 13C12-PCDD/Fs internal standards, the samples were extracted by Soxhlet extraction with toluene for 24 hours. The extract was concentrated and transferred to n-hexane, then treated with sulfuric acid (at least 3 times) in a separatory funnel. The hexane layer was rinsed with pure water and dried by passing through anhydrous sodium sulfate in a glass funnel. The solution was concentrated to about 2 mL and sequentially subjected to a multilayer silica gel packed glass column (1 cm of anhydrous sodium sulfate, 2 g of silica gel impregnated with AgNO3 (10% mass), 0.5 g of silica gel, 5 g of silica gel impregnated with sulfuric acid (50% mass), 0.5 g of silica gel, 2 g of silica gel impregnated with KOH (2% mass) and 0.5 g of silica gel in sequence) and eluted with 200 mL of hexane. The eluent was concentrated and further pass through activated carbon-impregnated silica gel column (1 g of anhydrous sodium sulfate, 1 g of activated carbon dispersed silica gel and 1 g of anhydrous sodium sulfate in sequence), and eluted with 30 mL hexane and 40mL 25% dichloromethane /hexane (v/v) as the first fraction. The second fraction was eluted with 50 mL toluene from the other end of the column containing PCDD/Fs. An aliquot of the second fraction was spiked with injection standards for PCDD/Fs instrumental analysis.

Table S1 A summary of PCDD/Fs in vegetation collected near MSWIs in other studiesa

| Country | Location | Time | Sample Collection | No.b |  |  | ∑PCDD/Fs |  |  | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | air | soil | vegetation | air (fg I-TEQ/m3) | soil (ng I-TEQ/kg dw) | vegetation (ng I-TEQ/kg dw) |  |
| Spain | Tarragonag | 1996 | 0.25,0.5,0.75, 1, 1.25, and 1,5 km from the stack in each of the four main wind directions |  | 24 | 24 |  | 1.08 (0.225-5.80) | 4.11 (0.15-62.09) | (Schuhmacher et al., 1997; Domingo et al., 2001a) |
|  |  | 1997 |  |  | 24 | 24 |  | 1.17 (0.11 - 3.88) | 0.23 (0.11 - 0.50) | (Schuhmacher et al., 1998b; Domingo et al., 2001a) |
|  |  | 1999 |  |  | 24 | 24 |  | 1.20 (0.15 -4.89 ) | 0.11 (0.04- 0.28) | (Domingo et al., 2001c) |
|  |  | 2007 |  | 8 |  | 8 | 12.04 (3.93-32.53) |  | 0.10 (0.05-0.17) | (Vilavert et al., 2009) |
|  |  | 2008 |  | 8 | 8 |  | 15.21 (7.13-31.42) | 0.64 (0.13-2.41) |  |  |
|  |  | 2009 |  | 8 |  | 8 | 8.59 (3.61-22.7) |  | 0.06 (0.04-0.11) | (Vilavert et al., 2012) |
|  |  | 2010 |  | 8 | 8 |  | 10.5 (6.95-22.3) | 0.58 (0.11-1.35) |  |  |
|  |  | 2013 |  | 8 |  | 8 | 3.56de (1.88-5.29) |  | 0.07 (0.04-0.10) | (Vilavert et al., 2015) |
|  |  | 2014 |  | 8 | 8 |  | 10.1df (6.95-22.3) | 0.63 (0.16-1.69) |  |  |
|  | Montcadag | 1996 | 0.1, 0.25, 0.5, 0.75, 1,1.5, 2 and 3 km from the stack in each of the three main wind directions |  |  | 24 |  | 6.91 (0.28-44.30) | 1.92 (1.07-3.05) | (Schuhmacher et al. 1998c; Domingo et al., 2000a; Schuhmacher and Domingo, 2006) |
|  |  | 1997 |  |  | 24 | 24 |  | 4.48 (0.15-29.27) | 1. 30 (0.75-1.95) | (Domingo et al., 1999; Domingo et al., 2000a; Schuhmacher and Domingo, 2006) |
|  |  | 1998 |  |  | 24 | 24 |  | 9.95 （0.06 -127） | 0.95 (0.40 -1.94) | (Domingo et al., 2001b; Domingo et al., 2000a; Schuhmacher and Domingo, 2006) |
|  |  | 2000 |  |  | 24 | 24 |  | 5.49 | 0.77 | (Nadal et al., 2002; Schuhmacher and Domingo, 2006) |
|  |  | 2001- 2002 |  | 7 |  | 24 | 100 |  | 0.34 | (Schuhmacher et al., 2006 ;Schuhmacher and Domingo, 2006) |
|  | San Adrià del Besòsg | 1998 | 0.25, 0.5, 0.75, 1, 1.5 and 3 km from the stack. |  | 24 | 24 |  | 12.24 (1.22 -34.28 ) | 0.70 (0.33-1.98 ) | (Schuhmacher et al., 2000) |
|  |  | 1999 |  |  | 24 | 24 |  | 14.41 (1.33 -54.23 ) | 0.97 (0.32- 2.52) | (Domingo et al., 2000b) |
|  |  | 2000 |  |  | 23 | 23 |  | 7.09 (0.41- 121.46) | 0.61 (0.22- 1.20) | (Domingo et al., 2002b) |
|  |  | 2001 |  |  |  | 20 |  |  | 0.66 (0.23- 1.43) | (Domingo et al., 2002a) |
|  | Constantícg | 1996 | 0.05, 0.1, 1.5, 2, 2.5, 3, 3.5 and 4 km in the main wind directions from the stack i |  | 40 | 40 |  | 1.68 (0.13-24.20 ) | 0.61 (0.24-1.22 ) | (Schuhmacher et al., 1998a) |
|  |  | 1998 |  |  | 40 | 40 |  | 1.59 （0.12 -17.20） | 0.31 (0.14 -2.01) | (Schuhmacher et al., 2002a) |
|  |  | 2000 |  |  |  | 40 |  |  | 0.32 (0.13- 0.65) | (Schuhmacher et al., 2002b) |
|  |  | 2004- 2005 |  |  | 40 | 40 |  | 1.14 (0.06-12.60) | 0.40 (0.03-1.57) | (Mari et al., 2007) |
|  |  | 2008 |  |  | 40 | 40 |  | 2.89 (0.07-50.6) | 1.27 (0.23-3.60) | (Vilavert et al., 2010) |
|  |  | 2009 |  |  | 30 |  |  | 0.75 (0.09-2.99) |  |  |
| China | Taiwanh | 2004 | the stack as the center of a circle within a radius of 3 to 5 km | 7 | 7 | 7 | 237e | 3.4 | 4.92g | (Wang et al., 2008) |
|  | Shenzhenh | 2013- 2014 | 1, 2, 3, 4 and 5 km in one main wind direction from the stack | 60 |  | 30 | 400d e |  | 4.87g | This study |
| a mean values are given, with the range of values in parentheses; b Number of samples; c hazardous waste incinerator; d fg WHO-TEQ/m3; e active sampler; f passive sampler; g herbage samples (Pipatherum paradoxum L.); h Banyan leaves; i only two samples collected at 4 km, 6 of 10 samples collected >4 km were adjacent to roads and streets which were influenced by the traffic as an important source of PCDD/Fs. |  |  |  |  |  |  |  |  |  |  |

# Table S2. PCDD/F Concentrations in Other Types of Commercial Food from Market (range, pg/g ww).

|  | Cereals |  |  |  | Fruits |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | rice (n=6) | rice flour (n=2) | wheat flour (n=2) |  | litchi (n=3) | peach (n=3) | longan (n=3) | strawberry (n=3) | orange (n=3) | apple (n=3) | banana (n=3) |
| 2378- T4CDD | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 12378- P5CDD | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123478-H6CDD | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123678- H6CDD | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.013 | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123789- H6CDD | n.d. | n.d.-0.011 | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 1234678-H7CDD | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| OCDD | n.d. | n.d. | n.d.-0.20 |  | n.d. | n.d.-0.045 | n.d. | n.d.-0.064 | n.d. | n.d. | n.d. |
| 2378- T4CDF | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.010 | n.d. | n.d. | n.d. | n.d. | n.d. |
| 12378-P5CDF | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.0080 | n.d. | n.d.-0.012 | n.d. | n.d. | n.d. |
| 23478-P5CDF | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123478- H6CDF | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.0072 | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123678- H6CDF | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. | n.d. |
| 123789- H6CDF | n.d. | n.d.-0.018 | 0.012-0.017 |  | n.d. | n.d.-0.0050 | n.d.-0.0033 | n.d. | n.d. | n.d. | n.d. |
| 234678- H6CDF | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.0081 | n.d. | n.d. | n.d. | n.d. | n.d. |
| 1234678- H7CDF | n.d. | n.d. | n.d. |  | n.d. | n.d.-0.018 | n.d.-0.023 | n.d. | n.d. | n.d. | n.d. |
| 1234789- H7CDF | n.d. | n.d.-0.017 | 0.014 |  | n.d. | n.d.-0.0050 | n.d. | n.d. | n.d. | n.d. | n.d. |
| OCDF | n.d. | n.d. | n.d.-0.054 |  | n.d.-0.015 | n.d.-0.029 | n.d. | n.d. | n.d. | n.d. | n.d. |
| Total (pg/g ww) | n.d. | n.d.-0.040 | 0.02-0.28 |  | n.d.-0.015 | 0.013.-0.13 | n.d.-0.023 | n.d.-0.064 | n.d. | n.d. | n.d. |
| TEQ (pg TEQ/g ww) | n.d. | n.d.-0.0030 | 0.0013-0.019 |  | n.d.-0.0000015 | 0.0013-0.0038 | n.d.-0.00033 | n.d.-0.00035 | n.d. | n.d. | n.d. |

n.d.: not detected.

# (Continued)

|  | Fruits |  | Milk (n=8) |  | Eggs |  |  |  |  | Meat |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | pear (n=3) |  |  |  | quail eggs (n=4) | duck eggs (n=4) | chicken eggs (n=4) |  | pork (n=8) | beef (n=8) | poultry (n=8) |
| 2378- T4CDD | n.d. |  | n.d. |  | n.d. | n.d. | n.d. |  | n.d. | n.d. | n.d. |
| 12378- P5CDD | n.d. |  | n.d.-0.076 |  | n.d.-0.035 | n.d.-0.11 | n.d.-0.066 |  | n.d.-0.11 | n.d.-0.36 | n.d.-0.11 |
| 123478-H6CDD | n.d. |  | n.d.-0.086 |  | n.d.-0.022 | n.d.-0.10 | n.d.-0.042 |  | n.d.-0.028 | n.d.-0.015 | n.d.-0.045 |
| 123678- H6CDD | n.d. |  | n.d.-0.10 |  | n.d. | 0.040-0.20.- | n.d.-0.16 |  | n.d. | n.d.-0.063 | n.d.-0.11 |
| 123789- H6CDD | n.d. |  | n.d-0.089 |  | n.d.-0.021 | 0.017-0.14 | n.d.-0.084 |  | n.d.-0.040 | n.d. | n.d.-0.072 |
| 1234678-H7CDD | n.d. |  | n.d.-0.063 |  | n.d.-0.053 | 0.10-1.5 | n.d.-0.16 |  | n.d. | n.d.-0.089 | n.d.-1.60 |
| OCDD | n.d.-0.024 |  | n.d.-0.11 |  | 0.14-0.29 | 0.62-15 | n.d.-0.35 |  | n.d.-2.74 | n.d.-1.7 | n.d.-26 |
| 2378- T4CDF | n.d. |  | n.d.-0.11 |  | n.d. | n.d.-0.11 | n.d.-0.10 |  | n.d. | n.d. | n.d.-0.28 |
| 12378-P5CDF | n.d. |  | n.d.-0.041 |  | n.d. | 0.034-0.094 | 0.030-0.10 |  | n.d. | n.d.-0.034 | n.d.-0.23 |
| 23478-P5CDF | n.d. |  | n.d.-0.086 |  | n.d.-0.062 | n.d.-0.10 | n.d.-0.15 |  | n.d. | n.d.-0.52 | n.d.-0.22 |
| 123478- H6CDF | n.d. |  | n.d.-0.098 |  | n.d. | n.d.-0.10 | n.d.-0.11 |  | n.d.-0.074 | n.d.-0.45 | n.d.-0.22 |
| 123678- H6CDF | n.d. |  | n.d.-0.083 |  | n.d.-0.035 | n.d.-0.094 | n.d.-0.21 |  | n.d.-0.092 | n.d.-0.56 | n.d.-0.11 |
| 123789- H6CDF | n.d. |  | n.d.-0.089 |  | n.d. | n.d. | n.d.-0.024 |  | n.d.-0.055 | n.d. | n.d.-0.036 |
| 234678- H6CDF | n.d. |  | n.d.-0.098 |  | n.d.-0.035 | n.d.-0.034 | n.d.-0.049 |  | n.d.-0.064 | n.d.-0.27 | n.d.-0.057 |
| 1234678- H7CDF | n.d. |  | n.d.-0.071 |  | n.d.-0.062 | n.d.-0.080 | n.d.-0.12 |  | n.d.-0.10 | n.d.-0.16 | n.d.-0.25 |
| 1234789- H7CDF | n.d. |  | n.d.-0.054 |  | n.d. | n.d.-0.029 | n.d. |  | n.d. | n.d. | n.d.-0.028 |
| OCDF | n.d. |  | n.d.-0.13 |  | n.d.-0.062 | n.d. | n.d.-0.10 |  | n.d. | n.d. | n.d.-1.02 |
| Total (pg/g ww) | n.d.-0.024 |  | n.d.-1.20 |  | 0.23-0.49 | 0.91-17 | 0.030-1.4 |  | n.d.-2.84 | n.d.-2.5 | 0.12-29 |
| TEQ (pg TEQ/g ww) | n.d.-0.0000071 |  | n.d.-0.14 |  | 0.000087-0.062 | 0.016-0.24 | 0.00091-0.15 |  | n.d.-0.12 | n.d.-0.64 | 0.000083-0.28 |

n.d.: not detected.

# (Continued)

|  | Fish and shrimp |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | bighead carp (n=3) | crucian (n=2) | mullet (n=3) | catfish (n=1) | bass (n=3) | grass carp (n=3) | tilapia (n=5) | marine fish (n=4) | spiral shell (n=3) | shrimp (n=3) |
| 2378- T4CDD | n.d.-0.053 | n.d.-0.029 | n.d.-0.078 | 0.062 | n.d. | n.d.-0.011 | n.d.-0.013 | n.d.-0.010 | n.d. | n.d. |
| 12378- P5CDD | 0.051-0.23 | 0.056-0.082 | 0.15-0.17 | 0.43 | 0.036-0.082 | 0.057-0.29 | n.d.-0.039 | n.d.-0.33 | n.d.-0.049 | n.d.-0.026 |
| 123478-H6CDD | 0.023-0.13 | n.d.-0.063 | 0.067-0.11 | 0.46 | n.d. | 0.023-0.085 | n.d.-0.017 | n.d.-0.033 | 0.025-0.035 | n.d-0.015. |
| 123678- H6CDD | 0.063-0.47 | n.d. | 0.084-0.10 | 0.86 | n.d.-0.041 | 0.040-0.16 | 0.033-0.043 | n.d.-0.061 | 0.091-0.11 | n.d.-0.082 |
| 123789- H6CDD | 0.028-0.42 | n.d. | 0.20-0.23 | 0.86 | n.d.-0.026 | 0.029-0.10 | n.d.-0.17 | n.d.-0.039 | 0.042-0.067 | n.d.-0.015 |
| 1234678-H7CDD | 0.21-1.2 | n.d. | 0.35-0.40 | 12 | n.d.-0.087 | 0.26-0.40 | 0.049-0.23 | n.d.-0.094 | 0.48-0.50 | n.d. |
| OCDD | 0.48-2.3 | n.d.-0.22 | 1.1-1.2 | 63 | 0.51-0.56 | 1.7-2.9 | 0.22-3.3 | n.d.-0.46 | 3.7-3.8 | 0.32-0.35 |
| 2378- T4CDF | n.d.-0.59 | 0.11-0.14 | 0.35-0.45 | 0.097 | n.d.-0.056 | 0.18-0.64 | n.d.-0.17 | n.d.-0.21 | 0.28-0.33 | n.d.-0.051 |
| 12378-P5CDF | 0.040-0.20 | 0.034-0.045 | 0.15-0.18 | 0.26 | 0.036-0.062 | 0.097-0.46 | n.d.-0.062 | n.d.-0.15 | 0.067-0.094 | 0.030-0.071 |
| 23478-P5CDF | 0.066-0.27 | 0.066-0.10 | 0.21-0.24 | 0.42 | 0.072-0.082 | 0.12-0.38 | n.d.-0.12 | n.d.-0.33 | 0.039-0.052 | n.d.-0.051 |
| 123478- H6CDF | n.d.-0.067 | n.d. | 0.042-0.10 | 0.20 | n.d. | 0.043-0.16 | n.d.-0.074 | n.d.-0.044 | n.d.-0.049 | n.d. |
| 123678- H6CDF | n.d.-0.067 | n.d. | n.d.-0.050 | 0.19 | n.d.-0.041 | n.d.-0.16 | n.d.-0.79 | n.d.-0.050 | n.d.-0.046 | n.d.-0.041 |
| 123789- H6CDF | n.d. | n.d. | 0.021-0.035 | 0.097 | n.d.-0.031 | n.d.-0.028 | n.d.-0.012 | n.d. | n.d.-0.028 | n.d. |
| 234678- H6CDF | n.d.-0.16 | n.d. | n.d.-0.025 | 0.14 | n.d. | 0.027-0.13 | n.d.-0.070 | n.d.-0.056 | 0.021-0.031 | n.d.-0.026 |
| 1234678- H7CDF | n.d.-0.10 | n.d. | n.d. | 0.28 | n.d. | n.d.-0.092 | n.d.-0.15 | n.d.-0.072 | n.d.-0.056 | n.d. |
| 1234789- H7CDF | n.d.-0.013 | n.d. | 0.011-0.015 | 0.15 | n.d. | n.d. | n.d.-0.017 | n.d. | n.d.-0.028 | n.d. |
| OCDF | n.d. | n.d. | n.d. | 1.2 | n.d. | n.d. | n.d.-0.066 | n.d. | n.d.-0.056 | n.d. |
| Total (pg/g ww) | 1.4-4.8 | 0.45-0.50 | 2.9-3.1 | 80 | 0.72-0.97 | 2.7-6.0 | 0.36-3.9 | 0.29-1.3 | 5.0-5.1 | 0.45-0.58 |
| TEQ (pg TEQ/g ww) | 0.13-0.47 | 0.087-0.16 | 0.31-0.42 | 1.06 | 0.065-0.12 | 0.15-0.57 | 0.0049-0.12 | 0.000087-0.49 | 0.074-0.13 | 0.018-0.031 |

n.d.: not detected.

# Table S3. Human Exposure to Dioxin via Vegetable Consumption in Other Studies.

| Country | Sampling time | Vegetables | Consumption rate | ∑PCDD/Fs | Probable Daily Intake | vegetable contribution % | Reference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| (region) |  |  | (g/day) | （pg WHO-TEQ/g ww） | pg WHO TEQ/ kg day |  |  |
| Finland | 1997-1999 | tomato, carrot, cucumber, onion, potato. | 239 | 0.0000031-0.0010 | 0.76 | 0.21% | (Kiviranta et al., 2004) |
| Spain | 2006 | lettuce, tomato, cauliflower, string bean, potato, lentil, haricot bean. | 263 | 0.010 (0.003-0.023) | 0.37 | 9.7 % | (Llobet et al., 2008) |
| Japan | 2001-2003 | pulse, green vegetables and other vegetables, mushroom and seaweeds. | 359-387 | 0.00010-0.020 | 0.44-0.56 | 1.0-7.7 % | (Sasamoto et al., 2006) |
| US | 1999-2002 | __ | __ | __ | 0.04-0.33 | 10.8a % | (Charnley and Doull, 2005) |
| France | 1999 2005 | __ | __ | 0.01 | 1.31 0.53 | 9a %, 6a % | (Tard et al., 2007) |
| Italy | __ | __ | 272.6 | 0.028 | 0.96 | 20a % | (Fattore et al., 2006) |
| China | 2011 | vegetables and potatoes | __ | 0.011-0.015 | 0.37 | 18.6%b | (Zhang et al. 2015) |
| China (Shenzhen) | 2014-2015 | See Table1 | 336.7 | 0.020 | 0.56 | 20.1% | This study |

a: vegetables and fruits; b: PCDD/Fs and dioxin like PCBs.

Figure S1. Prediction of the distribution of annual average concentration of PCDD/Fs by the Gaussian model, a stack of 80 m height, concentration unit: ìg TEQ/m3 (Environmental  Assessment  Center  of  Peking  University, 2003).

Figure S2. PCDD/F congener patterns in atmospheric samples, soil samples and vegetable samples grown in the vicinity of the MSWI.

References

Charnley, G., Doull, J., 2005. Human exposure to dioxins from food, 1999-2002. Food Chem. Toxicol. 43, 671-679.

Domingo, J.L., Bocio, A., Nadal, M., Schuhmacher, M., Llobet, J.M., 2002a. Monitoring dioxins and furans in the vicinity of an old municipal waste incinerator after pronounced reductions of the atmospheric emissions. J. Environ. Monit. 4, 395-399.

Domingo, J.L., Granero, S., Schuhmacher, M., 2000a. Assessment of the environmental impact of PCDD/Fs in the vicinity of a municipal waste incinerator: Congener profiles of PCDD/Fs in soil and vegetation samples. J. Environ. Sci. Health., Part A 35,1195-1209.

Domingo, J.L., Granero, S., Schuhmacher, M., 2001a. Congener profiles of PCDD/Fs in soil and vegetation samples collected near to a municipal waste incinerator. Chemosphere 43,517-524.

Domingo, J.L., Schuhmacher, M., Agramunt, M.C.,Llobet, J.M., Rivera, J., Muller, L., 2002b. PCDD/F levels in the neighbourhood of a municipal solid waste incinerator after introduction of technical improvements in the facility. Environ. Int. 28, 19-27.

Domingo, J.L., Schuhmacher, M., Granero, S., De Kok, H.A.M., 2001b. Temporal variation of PCDD/PCDF levels in environmental samples collected near an old municipal waste incinerator. Environ. Monit. Assess. 69,175-193.

Domingo, J.L., Schuhmacher, M., Llobet, J.M., Muller, L., Rivera, J., 2001c. PCDD/F concentrations in soil and vegetation in the vicinity of a municipal waste incinerator after a pronounced decrease in the emissions of PCDD/Fs from the facility. Chemosphere 43, 217-226.

Domingo, J.L., Schuhmacher, M., Meneses, M., Granero, S., Llobet, J.M., de Kok, H.A.M., 1999. Monitoring dioxins and furans near an old municipal solid waste incinerator: Temporal variation in vegetation. J. Environ. Sci. Health., Part A 34,165-181..

Domingo, J.L., Schuhmacher, M., Muller, L., Rivera, J., Granero, S., Llobet, J.M., 2000b. Evaluating the environmental impact of an old municipal waste incinerator: PCDD/F levels in soil and vegetation samples. J. Hazard. Mater. 76, 1-12.

Fattore, E., Fanelli, R., Turrini, A., di Domenico, A., 2006. Current dietary exposure to polychlorodibenzo-p-dioxins, polychlorodibenzofurans, and dioxin-like polychlorobiphenyls in Italy. Mol. Nutr. Food Res. 50,915-921.

Kiviranta, H., Ovaskainen, M.A.L., Vartiainen, T., 2004. Market basket study on dietary intake of PCDD/Fs, PCBs, and PBDEs in Finland. Environ. Int. 30,923-932.

Llobet, J.M., Marti-Cid, R., Castell, V., Domingo, J.L., 2008. Significant decreasing trend in human dietary exposure to PCDD/PCDFs and PCBs in Catalonia, Spain. Toxicol. Lett. 178,117-126.

Mari, M., Nadal, M., Ferre-Huguet, N., Schuhmacher, M., Borrajo, M.A., Domingo, J.L., 2007. Monitoring PCDD/Fs in soil and herbage samples collected near a hazardous waste incinerator: Health risks for the population living nearby. Hum. Ecol. Risk Assess.13,1255-1270.

Nadal, M., Agramunt, M.C., Schuhmacher, W., Domingo, J.L., 2002. PCDD/PCDF congener profiles in soil and herbage samples collected in the vicinity of a municipal waste incinerator before and after pronounced reductions of PCDD/PCDF emissions from the facility. Chemosphere 49,153-159.

Sasamoto, T., Ushio, F., Kikutani, N., Saitoh, Y.,Yamaki, Y., Hashimoto, T., Horii, S., Nakagawa, J.-i., Ibe, A., 2006. Estimation of 1999-2004 dietary daily intake of PCDDs, PCDFs and dioxin-like PCBs by a total diet study in metropolitan Tokyo, Japan. Chemosphere 64,634-641.

Schuhmacher, M., Agramunt, M.C., Rodriguez-Larena, M.C., Diaz-Ferrero, J., Domingo, J.L., 2002a. Baseline levels of PCDD/Fs in soil and herbage samples collected in the vicinity of a new hazardous waste incinerator in Catalonia, Spain. Chemosphere 46,1343-1350.

Schuhmacher, M., Domingo, J.J., Llobet, J.M., Muller, L., Sunderhauf, W., Jager, J., 1998a .Baseline levels of PCDD/Fs in vegetation samples collected in the vicinity of a new hazardous waste incinerator in Catalonia, Spain. Chemosphere 36,2581-2591.

Schuhmacher, M., Domingo, J.L., 2006. Long-term study of environmental levels of dioxins and furans in the vicinity of a municipal solid waste incinerator. Environ. Int. 32, 397-404.

Schuhmacher, M., Domingo, J.L., Llobet, J.M., Muller, L., Jager, J., 1997. Levels of PCDDs and PCDFs in grasses and weeds collected near a municipal solid waste incinerator. Sci. Total Environ. 201,53-62.

Schuhmacher, M., Domingo, J.L., Llobet, J.M., Sunderhauf, W., Muller, L., 1998b. Temporal variation of PCDD/F concentrations in vegetation samples collected in the vicinity of a municipal waste incinerator (1996-1997). Sci. Total Environ. 218, 175-183.

Schuhmacher, M., Domingo, J.L., Xifro, A., Granero, S., Llobet, J.M., de Kok, H.A.M., 1998c. Presence of dioxins and furans in vegetation samples collected in the neighbourhood of a municipal solid waste incinerator. J. Environ. Sci. Health., Part A 33, 195-212..

Schuhmacher, M., Granero, S., Rivera, J., Muller, L., Llobet, J.M., Domingo, J.L., 2000. Atmospheric deposition of PCDD/Fs near an old municipal solid waste incinerator: levels in soil and vegetation. Chemosphere 40, 593-600.

Schuhmacher, M., Jones, K.C., Domingo, J.L., 2006. Air-vegetation transfer of PCDD/PCDFs: An assessment of field data and implications for modeling. Environ.Pollut. 142,143-150.

Schuhmacher, M., Rodriguez-Larena, M.C., Agramunt, M.C., Diaz-Ferreo, J., Domingo, J.L., 2002b. Environmental impact of a new hazardous waste incinerator in Catalonia, Spain: PCDD/PCDF levels in herbage samples. Chemosphere 48, 187-193.

Tard, A., Gallotti, S., Leblanc, J.C., Volatier, J.L., 2007. Dioxins, furans and dioxin-like PCBs: Occurrence in food and dietary intake in France. Food Addit. Contam..24,1007-1017.

Vilavert, L., Nadal, M., Mari, M., Schuhmacher, M., Domingo, J.L., 2009. Modification of an environmental surveillance program to monitor PCDD/Fs and metals around a municipal solid waste incinerator. J. Environ. Sci. Health., Part A 44, 1343-1352.

Vilavert, L., Nadal, M., Mari, M., Schuhmacher, M., Domingo, J.L., 2010. Monitoring Temporal Trends in Environmental Levels of Polychlorinated Dibenzo-p-dioxins and Dibenzofurans: Results From a 10-Year Surveillance Program of a Hazardous Waste Incinerator. Arch. Environ. Contam. Toxicol. 59,521-531.

Vilavert, L., Nadal, M., Schuhmacher, M., Domingo, J.L., 2012.  Long-term monitoring of dioxins and furans near a municipal solid waste incinerator: human health risks. Waste Manage. Res. 30, 908-916.

Vilavert, L., Nadal, M., Schuhmacher, M., Domingo, J.L., 2015. Two Decades of Environmental Surveillance in the Vicinity of a Waste Incinerator: Human Health Risks Associated with Metals and PCDD/Fs. Arch. Environ. Contam. Toxicol. 69, 241-253.

Wang, J.B., Wang, M.S., Wu, E.M.Y., Chang-Chien, G.P., Lai, Y.C., 2008. Approaches adopted to assess environmental impacts of PCDD/F emissions from a municipal solid waste incinerator. J. Hazard. Mater. 152, 968-975.

Zhang, L., Yin, S.X., Wang, X.Y.,  Li, J.G., Zhao, Y.F., Li, X.W.,  Shen, H.T., Wu, Y.N., 2015. Assessment of dietary intake of polychlorinated dibenzo-p-dioxins and dibenzofurans and dioxin-like polychlorinated biphenyls from the Chinese Total Diet Study in 2011. Chemosphere 137, 178-184.