Supplementary Material

for

Age-dependent Human Elimination Half-lives of Dioxin-like Polychlorinated Biphenyls Derived from Cross-Sectional Biomonitoring Data in the General Population

Qun Gao1, Yujie Ben1, Zhaomin Dong2, and Jianying Hu1*

1MOE Laboratory for Earth Surface Process, College of Urban and Environmental Sciences, Peking University, Beijing 100871, China

2School of Space and Environment, Beihang University, Beijing 100191, China

Corresponding author:

Jianying Hu, College of Urban and Environmental Sciences, Peking University, Beijing 100871, China, TEL & FAX: 86-10-62765520, e-mail: hujy@urban.pku.edu.cn

Materials and Methods

Chemicals and Regents. 13C-labeled surrogate standards: WP-LCS (including 13C12-PCB77, 13C12-PCB81, 13C12-PCB105, 13C12-PCB114, 13C12-PCB118, 13C12-PCB123, 13C12-PCB126, 13C12-PCB156, 13C12-PCB157, 13C12-PCB167, 13C12-PCB169, 13C12-PCB189, 50 ng/mL), injection standards: WP-ISS (including 13C12-PCB70 and 13C12-PCB111, 13C12-PCB170,50 ng/mL), and calibration standard solutions: CS2, CS4, CS5, CS6 and CS7 of WP-CVs were obtained from Wellington Laboratories (Guelph, Canada). Hexane, dichloromethane and toluene were ultra residues analysis grade and purchased from J.T. Baker Co. Ltd. (Phillipsburg, USA). Sodium sulfates was pesticide analysis grade and purchased from ANPEL Laboratory Technologies Inc. (Shanghai, China). Silica gel and activated carbon dispersed silica gel were purchased from Kanto Chemical Co. Inc (Tokyo, Japan).

Analytical methods for blood samples and food samples

The analysis of dioxin-like PCBs of blood samples were performed according to Nakamura’s (2008) method and food samples were performed according to our previous study (Wan et al., 2005) method.

Blood samples (7g) were spiked with 13C12-labeled PCB congeners. The mixture samples were extracted using liquid-liquid extraction with 6 mL saturated ammonium sulfate, using 24 mL ethanol: n-hexane (v/v, 1/3). The layer of n-hexane was collected and the residue was extracted with n-hexane. The combined n-hexane extract was then rinsed with pure water and dried by passing through anhydrous sodium sulfate in a glass funnel. Weighting fat. The solution was concentrated to about 1 mL and sequentially subjected to a multilayer silica gel packed glass column and eluted with 100 mL of hexane. The eluent was concentrated and further pass through activated carbon-impregnated silica gel column and eluted with 3 mL hexane and 7 mL 25% dichloromethane /hexane (v/v) as the first fraction. The second fraction was eluted with 10 mL toluene from the other end of the column. The first fraction spiked with injection standards for four non-ortho PCBs instrumental analysis and the second fraction for eight mono-ortho PCBs instrumental analysis.

Food samples such as meat, fruits and vegetables were washed by distilled water, chopped and freeze-dried. Eggs, shrimp and spiral shell were shelled and freeze-dried. After spiked with a mixture of 13C12-PCB internal standards, the samples were extracted by Soxhlet extraction with toluene for 24 hours. The extract was concentrated and transferred to n-hexane, then treated with sulfuric acid in a separatory funnel. The hexane layer was rinsed with pure water and dried by passing through anhydrous sodium sulfate in a glass funnel. The solution was concentrated to about 1 mL and sequentially subjected to a multilayer silica gel packed glass column and eluted with 200 mL of hexane. The eluent was concentrated and further pass through activated carbon-impregnated silica gel column, and eluted with 30 mL hexane and 40mL dichloromethane /hexane (v/v, 1/3) as the first fraction. The second fraction was eluted with 50 mL toluene from the other end of the column containing PCBs. Similarly, the first fraction and the second fraction spiked with injection standards for non-ortho PCBs and mono-ortho PCBs instrumental analysis, respectively.

Instrumental analysis

Twelve dioxin-like PCBs were measured by an Agilent 6890 series high-resolution gas chromatograph (Agilent Technologies Inc., California, USA) interfaced with a JMS-800D Mass Spectrometer (JEOL Ltd., Tokyo, Japan) equipped with DB-5MS fused silica capillary column (60m length, 0.25 mm ID, 0.25μm film thickness, J&W Scientific, USA). The GC oven temperature program for 8 mono-ortho PCBs was set as follows: the initial temperature was 150 °C maintained for 1 min, which was increased to 185 °C at a rate of 20 °C/min, then the temperature was increased at a rate of 2 °C/min to 245 °C maintained for 3 min, and then the temperature was increased at a rate of 6 °C/min to achieve the final temperature of 290 °C, which was maintained for 5 min. The temperature program for the 4 non-ortho PCBs was: the initial temperature was 110 °C maintained for 0.5 min, which was increased to 180 °C at a rate of 22 °C/min, then the temperature was increased at a rate of 2 °C/min to 235 °C maintained for 4 min, and then the temperature was increased at a rate of 8 °C /min to achieve the final temperature of 310 °C, which was maintained for 12 min.

Quality assurance and quality control

For all analyzed samples, at least one of nine samples was subjected to laboratory blanks and ten field blank samples were also analyzed. The limit detection (LOD) for each compound was defined as three times the standard deviation of the analyzed blank concentrations. LODs of dioxin-like PCBs were 0.00322-0.00644 pg/g ww in human blood samples. LODs of dioxin-like PCBs in selected food samples were 0.000580-2.17 pg/g ww in eggs, 0.0274-3.43 pg/g ww in poultry, 0.0593-2.99 pg/g ww in porks, 0.0242-1.48 pg/g ww in beef, 0.00430-0.759 pg/g ww in fruits, 0.0195-2.07 pg/g ww in fish and shrimp, 0.0212-1.48 pg/g ww in cereal, 0.00408-0.902pg/g ww in milk, and 0.00309-2.26 pg/g ww in vegetables. The concentration of undetected congeners in all samples were set to zero. Average recoveries of 13C-labeled dioxin-like PCB internal standards was 81.5±38.5% in blood samples, and 82.0%±27.0% in food samples. All recoveries were within the specified reference range of US EPA Method 1613 (USEPA, 1994).

Table S1. The daily intake (pg TEQ/kg bw/day) of dioxin-like PCBs of Japanese general population from 2003 to 2016. The median values were uesed in this study.

| Year | Minimum | Maximum | Mean | Median |
| --- | --- | --- | --- | --- |
| 2003 | 0.053 | 3.9 | 0.66 | 0.46 |
| 2004 | 0.038 | 4.8 | 0.8 | 0.48 |
| 2005 | 0.045 | 3.1 | 0.53 | 0.34 |
| 2006 | 0.042 | 1.4 | 0.33 | 0.22 |
| 2007 | 0.027 | 4.2 | 0.45 | 0.25 |
| 2008 | 0.027 | 2.6 | 0.43 | 0.24 |
| 2009 | 0.017 | 4 | 0.48 | 0.21 |
| 2010 | 0.016 | 1.2 | 0.24 | 0.19 |
| 2011 | 0.019 | 1.7 | 0.34 | 0.17 |
| 2012 | 0.034 | 1.8 | 0.46 | 0.38 |
| 2013 | 0.024 | 1.1 | 0.19 | 0.09 |
| 2014 | 0.039 | 0.82 | 0.24 | 0.17 |
| 2015 | 0.05 | 0.74 | 0.22 | 0.14 |
| 2016 | 0.034 | 1 | 0.22 | 0.15 |

https://www.env.go.jp/chemi/dioxin/chosa/yasei.html [accessed 5 September 2018]

Table S2. The daily intake (pg/kg bw/day) of every congener of dioxin-like PCBs at each age.

| Age | PCB118 | PCB189 | PCB157 | PCB105 | PCB123 | PCB156 | PCB114 | PCB167 | PCB77 | PCB126 | PCB81 | PCB169 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 30.8 | 21.1 | 17.3 | 14.1 | 5.08 | 4.02 | 2.41 | 2.40 | 2.22 | 1.47 | 0.771 | 0.637 |
| 25 | 30.7 | 21.1 | 17.2 | 14.1 | 5.06 | 4.01 | 2.40 | 2.40 | 2.21 | 1.47 | 0.769 | 0.635 |
| 26 | 30.6 | 21.0 | 17.1 | 14.0 | 5.05 | 4.00 | 2.40 | 2.39 | 2.20 | 1.46 | 0.766 | 0.633 |
| 27 | 30.5 | 20.9 | 17.1 | 14.0 | 5.03 | 3.99 | 2.39 | 2.38 | 2.20 | 1.46 | 0.764 | 0.631 |
| 28 | 30.4 | 20.9 | 17.0 | 13.9 | 5.02 | 3.98 | 2.38 | 2.38 | 2.19 | 1.45 | 0.762 | 0.629 |
| 29 | 30.3 | 20.8 | 17.0 | 13.9 | 5.01 | 3.97 | 2.38 | 2.37 | 2.19 | 1.45 | 0.760 | 0.628 |
| 30 | 30.3 | 20.8 | 17.0 | 13.9 | 5.00 | 3.96 | 2.37 | 2.36 | 2.18 | 1.45 | 0.759 | 0.626 |
| 31 | 30.2 | 20.7 | 16.9 | 13.9 | 4.99 | 3.95 | 2.37 | 2.36 | 2.18 | 1.44 | 0.757 | 0.625 |
| 32 | 30.2 | 20.7 | 16.9 | 13.8 | 4.98 | 3.94 | 2.36 | 2.36 | 2.17 | 1.44 | 0.756 | 0.624 |
| 33 | 30.1 | 20.7 | 16.9 | 13.8 | 4.97 | 3.94 | 2.36 | 2.35 | 2.17 | 1.44 | 0.755 | 0.623 |
| 34 | 30.1 | 20.7 | 16.9 | 13.8 | 4.97 | 3.93 | 2.36 | 2.35 | 2.17 | 1.44 | 0.754 | 0.622 |
| 35 | 30.1 | 20.6 | 16.8 | 13.8 | 4.96 | 3.93 | 2.36 | 2.35 | 2.17 | 1.44 | 0.753 | 0.622 |
| 36 | 30.0 | 20.6 | 16.8 | 13.8 | 4.96 | 3.93 | 2.35 | 2.35 | 2.16 | 1.44 | 0.753 | 0.621 |
| 37 | 30.0 | 20.6 | 16.8 | 13.8 | 4.96 | 3.92 | 2.35 | 2.34 | 2.16 | 1.43 | 0.752 | 0.621 |
| 38 | 30.0 | 20.6 | 16.8 | 13.8 | 4.95 | 3.92 | 2.35 | 2.34 | 2.16 | 1.43 | 0.752 | 0.621 |
| 39 | 30.0 | 20.6 | 16.8 | 13.8 | 4.95 | 3.92 | 2.35 | 2.34 | 2.16 | 1.43 | 0.752 | 0.621 |
| 40 | 30.0 | 20.6 | 16.8 | 13.8 | 4.95 | 3.92 | 2.35 | 2.34 | 2.16 | 1.43 | 0.752 | 0.621 |
| 41 | 30.0 | 20.6 | 16.8 | 13.8 | 4.95 | 3.92 | 2.35 | 2.34 | 2.16 | 1.43 | 0.752 | 0.621 |
| 42 | 30.0 | 20.6 | 16.8 | 13.8 | 4.96 | 3.93 | 2.35 | 2.35 | 2.16 | 1.44 | 0.753 | 0.621 |
| 43 | 30.1 | 20.6 | 16.8 | 13.8 | 4.96 | 3.93 | 2.35 | 2.35 | 2.16 | 1.44 | 0.753 | 0.622 |
| 44 | 30.1 | 20.7 | 16.9 | 13.8 | 4.97 | 3.93 | 2.36 | 2.35 | 2.17 | 1.44 | 0.754 | 0.622 |
| 45 | 30.1 | 20.7 | 16.9 | 13.8 | 4.97 | 3.94 | 2.36 | 2.35 | 2.17 | 1.44 | 0.755 | 0.623 |
| 46 | 30.2 | 20.7 | 16.9 | 13.8 | 4.98 | 3.94 | 2.36 | 2.35 | 2.17 | 1.44 | 0.756 | 0.624 |
| 47 | 30.2 | 20.7 | 16.9 | 13.8 | 4.98 | 3.95 | 2.37 | 2.36 | 2.17 | 1.44 | 0.757 | 0.625 |
| 48 | 30.2 | 20.8 | 16.9 | 13.9 | 4.99 | 3.95 | 2.37 | 2.36 | 2.18 | 1.45 | 0.758 | 0.626 |
| 49 | 30.3 | 20.8 | 17.0 | 13.9 | 5.00 | 3.96 | 2.37 | 2.37 | 2.18 | 1.45 | 0.759 | 0.627 |
| 50 | 30.4 | 20.8 | 17.0 | 13.9 | 5.01 | 3.97 | 2.38 | 2.37 | 2.19 | 1.45 | 0.761 | 0.628 |

Table S3. The half-life of each dioxin-like PCB at each age.

| Age | PCB105 | PCB114 | PCB118 | PCB123 | PCB156 | PCB157 | PCB167 | PCB189 | PCB126 | PCB169 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 24 | 5.79 | 6.60 | 7.51 | 3.52 | 15.1 | 2.08 | 10.0 | 0.81 | 3.32 | 5.24 |
| 25 | 5.79 | 6.74 | 7.69 | 3.59 | 15.1 | 2.15 | 10.2 | 0.83 | 3.39 | 5.36 |
| 26 | 5.79 | 6.88 | 7.87 | 3.66 | 15.1 | 2.21 | 10.4 | 0.86 | 3.46 | 5.48 |
| 27 | 5.79 | 7.04 | 8.07 | 3.73 | 15.1 | 2.28 | 10.6 | 0.89 | 3.53 | 5.61 |
| 28 | 5.79 | 7.20 | 8.26 | 3.81 | 15.1 | 2.35 | 10.9 | 0.92 | 3.60 | 5.75 |
| 29 | 5.79 | 7.36 | 8.46 | 3.88 | 15.1 | 2.42 | 11.1 | 0.95 | 3.67 | 5.89 |
| 30 | 5.79 | 7.54 | 8.67 | 3.97 | 15.1 | 2.49 | 11.4 | 0.99 | 3.74 | 6.04 |
| 31 | 5.79 | 7.72 | 8.88 | 4.05 | 15.1 | 2.57 | 11.7 | 1.02 | 3.82 | 6.20 |
| 32 | 5.79 | 7.92 | 9.09 | 4.14 | 15.1 | 2.65 | 12.0 | 1.06 | 3.90 | 6.36 |
| 33 | 5.79 | 8.12 | 9.32 | 4.23 | 15.1 | 2.73 | 12.3 | 1.09 | 3.98 | 6.54 |
| 34 | 5.79 | 8.33 | 9.54 | 4.33 | 15.1 | 2.81 | 12.6 | 1.13 | 4.06 | 6.72 |
| 35 | 5.79 | 8.56 | 9.77 | 4.43 | 15.1 | 2.90 | 13.0 | 1.17 | 4.14 | 6.92 |
| 36 | 5.79 | 8.80 | 10.0 | 4.53 | 15.1 | 2.99 | 13.3 | 1.21 | 4.22 | 7.13 |
| 37 | 5.79 | 9.05 | 10.2 | 4.65 | 15.1 | 3.08 | 13.7 | 1.25 | 4.31 | 7.35 |
| 38 | 5.79 | 9.32 | 10.5 | 4.76 | 15.1 | 3.17 | 14.1 | 1.30 | 4.39 | 7.58 |
| 39 | 5.79 | 9.60 | 10.8 | 4.88 | 15.1 | 3.27 | 14.6 | 1.34 | 4.48 | 7.83 |
| 40 | 5.79 | 9.90 | 11.0 | 5.01 | 15.1 | 3.37 | 15.0 | 1.39 | 4.57 | 8.10 |
| 41 | 5.79 | 10.2 | 11.3 | 5.15 | 15.1 | 3.47 | 15.5 | 1.44 | 4.67 | 8.38 |
| 42 | 5.79 | 10.6 | 11.6 | 5.29 | 15.1 | 3.57 | 16.0 | 1.49 | 4.76 | 8.69 |
| 43 | 5.79 | 10.9 | 11.8 | 5.44 | 15.1 | 3.68 | 16.6 | 1.54 | 4.86 | 9.02 |
| 44 | 5.79 | 11.3 | 12.1 | 5.60 | 15.1 | 3.79 | 17.2 | 1.59 | 4.95 | 9.38 |
| 45 | 5.79 | 11.7 | 12.4 | 5.78 | 15.1 | 3.91 | 17.8 | 1.64 | 5.05 | 9.76 |
| 46 | 5.79 | 12.2 | 12.7 | 5.96 | 15.1 | 4.03 | 18.5 | 1.70 | 5.16 | 10.2 |
| 47 | 5.79 | 12.7 | 13.0 | 6.15 | 15.1 | 4.15 | 19.3 | 1.76 | 5.26 | 10.6 |
| 48 | 5.79 | 13.2 | 13.4 | 6.36 | 15.1 | 4.28 | 20.1 | 1.82 | 5.37 | 11.1 |
| 49 | 5.79 | 13.8 | 13.7 | 6.58 | 15.1 | 4.41 | 21.0 | 1.88 | 5.47 | 11.7 |
| 50 | 5.79 | 14.4 | 14.0 | 6.81 | 15.1 | 4.54 | 22.0 | 1.95 | 5.59 | 12.3 |

References

Nakamura, T., Nakai, K., Matsumura, T., Suzuki, S., Saito, Y., Satoh, H., 2008. Determination of dioxins and polychlorinated biphenyls in breast milk, maternal blood and cord blood from residents of Tohoku, Japan. Sci. Total Environ. 394 (1), 39-51.

Wan, Y., Hu, J.Y., Yang, M., An, L.H., An, W., Jin, X.H., Hattori, T., Itoh, M., 2005. Characterization of trophic transfer for polychlorinated dibenzo-p-dioxins, dibenzofurans, non- and mono-ortho polychlorinated biphenyls in the marine food web of Bohai Bay, North China. Environ. Sci. Technol. 39(8), 2417-2425.

US EPA (U.S. Environmental Protection Agency), 1994. Method 1613: tetra-through octa-chlorinated dioxins and Furans by isotope dilution HRGC/HRMS. Office of water and analysis Division. Washington, DC.