Supplementary material for

Simultaneous determination of primary and secondary phthalate monoesters in Taihu Lake: Exploration of sources

Analysis of metabolites of PAEs

An Acquity Ultra Performance LC with a Waters Acquity UPLC BEH C18 column (100 × 2.1 mm, 1.7 μm particle size) (Waters, Milford, MA, USA) was used to separate the primary and secondary phthalate monoesters. The column was maintained at 40 °C and a flow rate of 0.3 mL/min, and the injection volume was 5 μL. Methanol (A) and water containing 0.1% acetic acid (B) were used as mobile phases: 10% A was increased to 40% in 0.5 min, and then to 50% in 1.5 min, and then to 60% in 3.5 min, and then to 70% in 4.5 min, and then to 80% in 7 min, and then to 100% in 8.5 min and kept for 1.5 min, followed by a decrease to initial conditions of 10% in 12 min to allow for equilibration. A Premier XE tandem quadrupole mass spectrometer equipped with a Z-Spray ionization (ESI) source (Waters, Milford, MA, USA) was used for determining the phthalate primary and secondary monoesters and operated in the negative ion mode. The optimized conditions for LC-MS-MS were listed in Supplementary Table 1. The solution was dried under a gentle stream of nitrogen and redissolved with 200 μL of methanol prior to analysis of UPLC-MS/MS (Waters, Milford, Massachusetts, U.S.). The detailed information on mass spectrometry (MS) parameters and multiple reaction monitoring (MRM) conditions is shown in Supplementary Table 1.

Analytical method of PAEs.

Identification and quantification of PAEs were performed by a gas chromatograph-mass spectrometer (GC-MS) (Agilent 6890N) equipped with 5975C mass spectrometer (Agilent Technologies) in EI (electron impact) mode. A HP-5MS capillary column (30 cm × 0.25 mm × 0.25 μm film thickness; J&W Scientific) were used for separating PAEs. A splitless injector was used at 250°C. The temperature program increased from 70°C (4 min) to 160°C at 20°C/min, to 230°C at 2°C/min, and then to 300°C (10 min) at a rate of 30°C/min. The ion source temperature was maintained at 280°C, and helium was as the carrier gas at a constant flow rate of 5 mL/min. The ion used for quantification of the phthalates except for DMP was m/z 149. The ion for DMP was m/z163, and each molecular ion peak was used for confirmation. Recovery tests were conducted by analyzing lake water samples which were spiked with 100 ng/L for PAEs (n = 3). Recoveries were calculated by subtracting background concentrations in nonspiked samples from spiked samples according to the following equation.

Recovery (%) = (Observed – Background) / Spiked Concentration) ×100%;

where “Observed” is the “determined concentration of a PAE in the spiked sample”; “Background” is the determined background concentration of a PAE in nonspiked sample. The average recoveries of PAEs were 76-110% in the lake water, and LODs were from 0.001 μg/L (DnOP) to 0.08 μg/L (DiBP).

Supplementary Table 1. General information of the target chemical

| Chemicals | CAS number. | Formula | Structure |
| --- | --- | --- | --- |
| phthalates |  |  |  |
| Dimethyl phthalate (DMP) | 131-11-3 | C10H10O4 |  |
| Diethyl phthalate(DEP) | 84-66-2 | C12H14O4 |  |
| Di-n-butyl phthalate(DnBP) | 84-74-2 | C16H22O4 |  |
| Di-iso-butyl phthalate (DiBP) | 84-69-5 | C16H22O4 |  |
| Di-(2-ethylhexyl) phthalate (DEHP) | 117-81-7 | C24H38O4 |  |
| Di-n-octyl phthalate(DnOP) | 117-84-0 | C24H38O4 |  |
| Butyl benzyl phthalate(BBP) | 85-68-7 | C19H20O4 |  |
| Di-iso-nonyl phthalate(DiNP) | 28553-12-0 | C26H42O4 |  |
| phthalate monoesters |  |  |  |
| monomethyl phthalate(MMP) | 4376-18-5 | C9H8O4 |  |
| monoethyl phthalate(MEP) | 2306-33-4 | C10H10O4 |  |
| mono-iso-butyl phthalate(MiBP) | 30833-53-5 | C12H14O4 |  |
| mono-n-butyl phthalate(MnBP) | 131-70-4 | C12H14O4 |  |
| mono-(2-ethylhexyl) phthalate(MEHP) | 4376-20-9 | C16H22O4 |  |
| monobenzyl phthalate (MBzP) | 2528-16-7 | C15H12O4 |  |
| mono-iso-nonyl phthalate (MiNP) | 297182-83-3 | C17H24O4 |  |
| mono-n-octyl phthalate (MnOP) | 5393-19-1 | C16H22O4 |  |
| Secondary phthalate monoesters |  |  |  |
| mono (3-carboxypropyl) phthalate (MCPP) | 66851-46-5 | C12H12O6 |  |
| mono (2-ethyl-5-hydroylhexyl) phthalate (MEHHP) | 40321-99-1 | C16H22O5 |  |
| mono (2-ethyl-5-oxo-hexyl) phthalate (MEOHP) | 40321-98-0 | C16H20O5 |  |
| mono (2-ethyl-5-carboxypentyl) phthalate (MECPP) | 40809-41-4 | C16H20O6 |  |
| mono-(4-methyl-7-carboxyheptyl) phthalate(7cx-MMeHP) | 936022-02-5 | C17H22O6 |  |

Supplementary Table 2. Optimized conditions for LC-MS-MS.

| Compounds | MRM transition | Cone voltage (V) | Collision energy (eV) |
| --- | --- | --- | --- |
| MMP | 177.8>106.8 | 20 | 10 |
|  | 177.8>76.8 | 20 | 18 |
| MMP-13C4 | 182.8>109 | 20 | 10 |
|  | 182.8>79 | 20 | 19 |
| MEP | 192.8>76.7 | 19 | 16 |
|  | 192.8>120.8 | 19 | 12 |
| MEP-13C4 | 196.7>79 | 19 | 16 |
|  | 196.7>122.8 | 20 | 10 |
| MCPP | 251.1>103 | 10 | 10 |
|  | 251.1>165 | 10 | 12 |
| MCPP-13C4 | 255.1>103 | 10 | 20 |
|  | 255.1>168.9 | 10 | 12 |
| MiBP | 220.9>76.8 | 21 | 16 |
|  | 220.9>70.8 | 21 | 12 |
| MnBP | 220.9>76.8 | 21 | 16 |
|  | 220.9>70.8 | 21 | 12 |
| MnBP-13C4 | 224.9>76.8 | 21 | 20 |
|  | 224.9>150.8 | 20 | 10 |
| MBzP | 255>182.9 | 21 | 12 |
|  | 255>76.8 | 21 | 20 |
| MBzP-13C4 | 259>106.7 | 20 | 14 |
|  | 259>76.6 | 20 | 24 |
| MEHHP | 293.1>120.5 | 28 | 19 |
|  | 293.1>142.6 | 25 | 13 |
| MEHHP-13C4 | 297.1>123.8 | 25 | 20 |
|  | 297.10>144.8 | 25 | 16 |
| MEOHP | 291.1>120.6 | 25 | 19 |
|  | 291.1>142.6 | 25 | 13 |
| MECPP | 307.1>158.8 | 15 | 14 |
|  | 307.10>120.8 | 15 | 24 |
| MECPP-13C4 | 311.1>159.8 | 15 | 12 |
|  | 311.1>112.8 | 15 | 30 |
| MEHP | 277.1>133.8 | 25 | 16 |
|  | 277.1>126.8 | 25 | 16 |
| MEHP-13C4 | 281.1>136.6 | 26 | 16 |
|  | 281.1>126.6 | 26 | 16 |
| MnOP | 277.1>126.8 | 25 | 16 |
|  | 277.1>76.8 | 25 | 16 |
| MoC6 | 249.1>76.8 | 25 | 18 |
|  | 249.1>205.1 | 25 | 12 |
| MiNP | 291>76.7 | 40 | 20 |
|  | 291>140.8 | 40 | 16 |
| MiNP-13C4 | 295.1>140.9 | 30 | 22 |
|  | 295.1>78.8 | 30 | 30 |
| 7cx-MMeHP | 321.3>172.8 | 25 | 12 |
|  | 321.3>76.7 | 25 | 40 |
| 7cx-MMeHP-d4 | 325.3>172.8 | 25 | 12 |
|  | 325.3>80.7 | 25 | 40 |

Common MS parameters: capillary voltage, 3.2 kV (ESI-); source temperature, 110°C; desolvation temperature, 450°C; source gas flow, 50 L/h; and desolvation gas flow, 800 L/h.

Supplementary Table 3. Concentrations of phthalates and their primary and secondary phthalate monoesters in the Taihu Lake.

|  |  | Phthalate |  |  |  |  | Primary and secondary phthalate monoesters |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Sites* | DMP | DEP | DiBP | DnBP | DEHP | MMP | MEP | MiBP | MnBP | MEHP |
| South Region | S1 S2 S3 S4 S5 S6 S7 S8 S9 S10 S11 S12 S13 S14 S15 S16 S17 S18 S19 S20 S21 S22 S23 | 3.151 <LOD <LOD 13.59 5.894 5.149 <LOD <LOD 7.904 6.931 16.86 <LOD <LOD <LOD 10.02 9.752 1.016 7.308 34.69 1.798 6.375 <LOD 5.341 | 13.82 8.137 8.146 13.49 6.552 10.17 3.727 0.255 9.895 11.93 17.12 3.073 5.968 5.136 7.314 1.201 <LOD 7.962 19.07 15.85 14.14 <LOD 0.923 | 106.9 102.2 111.4 94.98 74.89 280.4 47.52 43.61 85.33 277.2 425.7 108.2 193.4 290.7 214.9 202.3 49.62 18.54 149.2 104.7 115.2 211.8 68.89 | 120.5 146.8 195.8 159.4 355.4 389.5 73.5 103.3 68.53 168.0 443.1 324.8 49.92 156.7 254.9 304.6 153.2 56.47 265.6 247.1 141.1 152.1 192.4 | 39.12 311.9 91.41 135.1 245.6 79.02 198.9 30.52 43.57 305.8 528.2 95.45 478.2 <LOD 282.4 13.18 <LOD 42.60 983.6 134.4 217.0 <LOD 125.4 | 34.7 39.4 26.7 23.0 25.6 36.4 38.4 26.1 20.0 36.9 44.9 52.9 46.3 43.7 40.7 35.1 42.9 41.0 44.9 51.7 42.3 28.7 58.1 | <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD 6.71 6.88 <LOD <LOD <LOD <LOD <LOD <LOD <LOD <LOD | 12.2 17.7 10.3 7.78 7.70 8.01 27.7 7.40 12.7 10.3 19.5 14.7 6.81 4.79 17.1 9.75 6.97 6.30 9.02 10.3 6.67 9.87 25.3 | 7.66 17.2 8.13 10.8 20.3 17.0 23.5 13.4 8.27 13.1 40.8 23.5 9.74 13.4 22.7 21.1 17.1 8.55 6.82 16.7 5.51 12.9 48.7 | 6.74 9.56 9.03 6.22 5.18 10.3 12.5 10.6 56.3 12.4 7.47 14.0 12.4 24.6 9.64 12.0 11.7 17.8 5.87 7.81 8.32 22.1 16.6 |
| Estuary | S-e N-e1 N-e2 N-e3 N-e4 N-e5 N-e6 N-e7 | 18.71 122.47 203.83 33.17 109.1 94.24 117.59 133.78 | 16.79 151.2 374.9 31.51 59.57 139.3 7.786 146.3 | 10.48 94.82 167.8 56.15 44.32 94.89 138.9 127.5 | 12.35 695.9 187.1 205.3 116.0 213.1 186.2 1228 | 62.85 338.0 303.7 164.5 <LOD 278.1 4.237 <LOD | 35.9 61.3 47.2 48.1 45.2 52.1 74.9 25.8 | <LOD <LOD <LOD <LOD <LOD 39.5 15.5 4.24 | 9.59 23.1 15.2 13.2 8.99 30.7 26.7 6.21 | 24.0 42.8 20.1 15.7 16.8 60.0 185 12.8 | 51.5 20.7 43.0 32.4 34.1 112 42.9 12.7 |
| Gonghu Bay | N4 N5 N11 N12 N13 N14 N15 N16 | 112.1 17.53 125.19 90.81 140.5 83.27 68.17 10.62 | 261.8 35.57 124.3 108.4 142.4 164.6 129.1 100.1 | 125.5 77.82 100.2 406.3 64.38 85.93 35.39 63.35 | 375.1 600.7 338.4 1151 366.5 538.7 120.5 224.3 | 338.1 202.9 311.7 90.04 277.0 279.4 178.7 142.3 | 45.4 62.2 40.9 65.9 74.1 38.9 39.4 70.3 | <LOD <LOD <LOD 8.76 <LOD <LOD <LOD <LOD | 14.3 18.9 14.5 44.3 16.0 13.3 9.86 12.0 | 63.6 87.7 30.5 32.8 16.2 24.7 32.1 20.1 | 14.4 6.77 19.5 15.9 13.1 11.2 9.39 19.1 |
| Zhushan Bay | N1 N2 N6 N7 N8 N17 N18 N19 N20 N21 N22 N23 N24 N25 | 37.88 16.86 28.78 28.27 14.50 41.00 54.17 42.41 49.34 32.13 48.81 54.51 53.01 24.36 | 85.35 22.01 58.02 27.60 2.493 54.63 10.87 44.98 37.67 23.55 60.87 24.02 28.54 4.232 | 1238 1221 967.7 630.5 99.07 1009 138.5 1063 873.3 1040 568.9 491.6 58.77 141.3 | 1332 1046 1099 663.4 1651 859.2 529.9 1167 870.6 880.5 668.4 364.8 91.27 3384 | <LOD <LOD <LOD 122.6 938.9 <LOD 247.5 48.13 100.6 67.85 <LOD 27.78 <LOD 209.7 | 47.3 37.4 61.7 34.4 31.5 144.9 181.7 56.7 62.5 88.3 60.9 55.7 66.1 53.3 | 6.99 7.06 <LOD 7.57 <LOD 9.38 9.26 5.62 10.7 5.33 <LOD <LOD <LOD <LOD | 18.0 39.1 29.9 31.7 91.5 35.9 29.7 9.65 29.8 11.3 11.9 15.0 23.0 13.8 | 50.7 36.1 62.6 38.8 463 78.8 204 32.1 52.4 24.3 23.7 39.5 25.8 37.8 | 25.6 46.7 41.0 107 54.0 115 69.3 28.1 40.4 35.0 53.2 40.6 63.5 19.1 |
| Meiliang Bay | N3 N9 N10 N26 N27 N28 N29 N30 N31 N32 N33 N34 N35 | 17.73 13.17 18.10 13.23 21.20 19.71 28.58 22.84 14.74 17.58 12.79 45.02 23.46 | 4.884 11.37 10.65 24.06 8.553 10.85 19.88 3.018 6.774 9.868 9.314 22.76 9.718 | 301.7 19.06 199.3 126.1 182.8 272.2 214.2 18.08 86.18 52.21 101.7 45.85 59.96 | 1369 40.28 494.9 228.9 367.9 160.9 458.9 116.7 1779 419.1 2209 707.4 592.2 | 433.4 389.4 273.7 240.6 136.4 343.4 510.5 106.8 69.49 508.7 868.2 648.1 87.42 | 44.7 48.6 45.5 45.0 48.0 43.9 36.3 58.7 53.7 56.6 80.2 69.5 92.7 | 10.6 <LOD <LOD <LOD <LOD <LOD <LOD 8.45 <LOD <LOD 10.9 13.0 8.45 | 30.5 15.7 15.6 12.0 22.0 18.6 47.1 20.6 33.8 15.1 31.8 33.9 63.3 | 72.6 12.2 11.3 15.9 9.66 16.9 83.1 6.17 145 19.0 32.1 47.8 75.9 | 59.1 22.3 25.9 17.9 15.0 23.2 249 19.3 13.2 25.4 43.0 92.2 66.4 |

*see Fig.1

Supplementary Fig. 1. Relationship between DiBP and DnBP.